# ESP32 Cabinet Project – Settings Cheat Sheet

## 1) LED Row — Brightness & Timers

What controls LED brightness

LED1 brightness comes from the lowest (dimmest) active source among SW2–SW5.

If no drawers are open, LED1 falls back to the SW1 baseline brightness.

LED2 brightness simply follows SW1.

Brightness is on a 0–255 scale (≈0–100%).

Quick picks:
25% → 64    50% → 128    75% → 191    100% → 255

Where to edit brightness levels
These values appear near the top of your sketch:

// Baselines when SW1 is ON (door)

const int SW1_LED1_LEVEL = 50;   // LED1 when only SW1 is ON

const int SW1_LED2_LEVEL = 74;   // LED2 follows SW1

// Drawer levels (used when drawers are open)

const int SW2_LEVEL = 25;

const int SW3_LEVEL = 90;

const int SW4_LEVEL = 150;

const int SW5_LEVEL = 255;

How to change LED1 brightness

When only SW1 is ON → change SW1_LED1_LEVEL.

When a drawer switch is ON → change SW2_LEVEL, SW3_LEVEL, SW4_LEVEL, SW5_LEVEL.

Note: If multiple drawers are open, LED1 uses the dimmest value.

How to change LED2 brightness

// LED2 follows SW1

const int SW1_LED2_LEVEL = 74;

// Example: set LED2 ≈ 75%

// const int SW1_LED2_LEVEL = 191;

Courtesy Floor (LED1 only)
This sets the minimum brightness for LED1 when the door (SW1) or SW2 first turns ON:

const int LED1_COURTESY_LEVEL = (int)(255 * 0.03f + 0.5f);  // ≈3%

Examples:
• 5% → 13
• 10% → 26
If your regular brightness is lower than this during courtesy mode, LED1 will still show at least the courtesy floor.

⭐ LED Timers (NEW)
Your LEDs have two timer types you can adjust:

A) LED Auto-Off Timer (LED1 + LED2)
This is the timer that turns an LED OFF after being ON continuously:

const unsigned long LED_AUTO_OFF_MS = 4UL * 60UL * 1000UL;  // 4 minutes

Change it to:
• 6 minutes → 6UL * 60UL * 1000UL
• 10 minutes → 10UL * 60UL * 1000UL
• Disable auto-off → set to 0

B) LED1 Courtesy Timer (LED1 only)
Controls how long LED1 stays at the courtesy brightness after SW1 or SW2 triggers it:

const unsigned long LED1_COURTESY_MS = 4UL * 60UL * 1000UL;  // 4 minutes

Change to:
• 6 minutes → 6UL * 60UL * 1000UL
• 1 minute → 1UL * 60UL * 1000UL
• Disable courtesy → 0

Optional: Fade Feel & Polarity

const unsigned long FADE_TIME_MS_LED1 = 2000;

const unsigned long FADE_TIME_MS_LED2 = 2000;

const bool INVERT = false;   // Set true if LED driver is active-LOW

## 2) Switch Row (SW1–SW5)

OLED row example:
SW1:ON  SW2:OFF  SW3:ON  SW4:OFF  SW5:OFF
This row shows the live state of all switches.
Because the switches are wired active-LOW, the OLED will show:
• ON → switch pin is LOW (pressed / door open / drawer open)
• OFF → switch pin is HIGH (released / door closed / drawer closed)

⭐ Switch Pins (hardware connections)
(Do not modify these unless you physically rewire the cabinet.)

const int SW1_PIN = 4;    // Door

const int SW2_PIN = 19;   // Top drawer above door

const int SW3_PIN = 13;   // Drawer behind door

const int SW4_PIN = 14;   // Drawer behind door

const int SW5_PIN = 27;   // Drawer behind door

⭐ Switch Logic Interpretation (important)
Right now, each switch uses active-LOW logic:

bool s1 = (digitalRead(SW1_PIN) == HIGH); // HIGH = released (OFF), LOW = pressed (ON)

Meaning:
• LOW (0V) → ON
• HIGH (3.3V) → OFF
If any switch acts backwards on the screen (e.g., shows ON when it should be OFF), then invert its interpretation.

⭐ How to flip a switch that reacts backward
(You normally won’t need this, but it’s good to know.)
Current logic:

bool s1 = (digitalRead(SW1_PIN) == HIGH);

To reverse it, change to:

bool s1 = (digitalRead(SW1_PIN) == LOW);

Example — flipping only SW3:

bool s3 = (digitalRead(SW3_PIN) == LOW);

⭐ Edge Detection (advanced)
Your system uses edges (changes) from SW1 & SW2 for:
• LED1 courtesy timer
• LED1 lockout reset
• Door-open events
• Drawer-open timer resets
Example:

bool s1FallingEdge = (prevS1 && !s1);  // OFF → ON

bool s1RisingEdge  = (!prevS1 && s1);  // ON → OFF

If you ever flip the logic of a switch, edge definitions must follow the same interpretation.
(Your current setup already matches active-LOW wiring correctly.)

⭐ Summary (Switch Row behavior)
• Switches are active-LOW
• OLED shows ON/OFF based on your defined logic
• SW1 and SW2 directly influence LED courtesy-timer behavior
• All switches contribute to determining LED1 brightness
• SW1 is also used for manual fan activation (3-sec press)

## 3) Fan Speed / RPM Row

OLED row example:
SPD: 50%  RPM:1234  T:01:45
• SPD: 50% → current fan PWM duty (0–100%).
• RPM:1234 → smoothed tach reading from the fan.
• T:01:45 → manual-fan timer only shown while manual mode is active (driven by fanOffAtMs and the manual 6-minute curve).

⭐ Humidity-Based Fan Trigger Points
The fan normally runs automatically from humidity, using this block:

if (h >= 82.0f)      { humidityDuty = 255; }

else if (h >= 80.0f) { humidityDuty = 191; }

else if (h >= 78.0f) { humidityDuty = 128; }

else if (h >= 75.0f) { humidityDuty = 64;  }

else if (h >= 74.0f) { humidityDuty = 26;  }

else if (h >= 72.0f) { humidityDuty = fanWasOn ? 26 : 0; }

else                 { humidityDuty = 0;   }

Rough percent equivalents (0–255 scale):
• 255 ≈ 100%
• 191 ≈ 75%
• 128 ≈ 50%
• 64 ≈ 25%
• 26 ≈ 10%

How to adjust behavior:
• To make the fan start earlier, lower the humidity thresholds.
Example: start strong fan at 78% instead of 82%:
if (h >= 78.0f)      { humidityDuty = 255; }
else if (h >= 76.0f) { humidityDuty = 191; }
...
• To make the fan less aggressive, lower the duty values.
Example: make the top stage ≈75% instead of 100%:
if (h >= 82.0f)      { humidityDuty = 191; }   // 75% instead of 100%
else if (h >= 80.0f) { humidityDuty = 128; }   // 50%
...
• To keep fan from chattering ON/OFF around a threshold, notice this line:
else if (h >= 72.0f) { humidityDuty = fanWasOn ? 26 : 0; }
That creates a little hysteresis:
• If fan was already ON, it keeps running down to 72% at a low speed.
• If fan was OFF, it won’t suddenly turn on at 72% unless a higher stage triggers it.

⭐ Manual Fan Start (Hold SW1 ~3 seconds)
When you hold SW1 for 3 seconds, the code starts a 6-minute fan curve:

const unsigned long FAN_MANUAL_RAMP_MS  = 1UL * 60UL * 1000UL;  // 1 minute

const unsigned long FAN_MANUAL_HOLD_MS  = 4UL * 60UL * 1000UL;  // 4 minutes

const unsigned long FAN_MANUAL_DECEL_MS = 1UL * 60UL * 1000UL;  // 1 minute

// Total manual time:

// FAN_MANUAL_TOTAL_MS =

//    (FAN_MANUAL_RAMP_MS + FAN_MANUAL_HOLD_MS + FAN_MANUAL_DECEL_MS);

Shape of the curve:
1. Ramp up (1 min): from MANUAL_MIN_PCT → MANUAL_MAX_PCT
2. Hold (4 min): at MANUAL_MAX_PCT
3. Decel (1 min): from MANUAL_MAX_PCT → MANUAL_MIN_PCT, then OFF

const float MANUAL_MIN_PCT = 5.0f;   // bottom of manual curve

const float MANUAL_MAX_PCT = 75.0f;  // top of manual curve

So you can tune:
• Softer / quieter manual mode → reduce MANUAL_MAX_PCT.
• Gentle breeze only → lower both MANUAL_MIN_PCT and MANUAL_MAX_PCT.

⭐ Minimum Running Duty (stall protection — in depth)

const int  FAN_MIN_RUN_DUTY   = 25;   // ~10% duty

const unsigned long FAN_KICK_MS = 400;

On the 0–255 scale:
• FAN_MIN_RUN_DUTY = 25 → about 10% PWM
This prevents the fan from trying to run at uselessly low duty cycles.
There is also kick-start logic that briefly runs the fan at full power to overcome static friction.

🔧 How to choose a good FAN_MIN_RUN_DUTY for your fan
1. Find the “stall point” by testing low duties.
2. Set FAN_MIN_RUN_DUTY slightly above that value.
3. Re-test humidity stages so your lowest active duty is ≥ FAN_MIN_RUN_DUTY.

## 4) Humidity Control Row

Your cabinet automatically adjusts the fan speed based on humidity readings from the DHT22 sensor.
The OLED indirectly reflects this by showing:
SPD: 50%   RPM:1234   T:01:45
(Where SPD is the humidity-driven or manual fan duty.)

⭐ Humidity Trigger Table (Fan Speed Based on RH%)
This block determines WHEN the fan turns on and HOW FAST it runs:

if (h >= 82.0f)      { humidityDuty = 255; }

else if (h >= 80.0f) { humidityDuty = 191; }

else if (h >= 78.0f) { humidityDuty = 128; }

else if (h >= 75.0f) { humidityDuty = 64;  }

else if (h >= 74.0f) { humidityDuty = 26;  }

else                 { humidityDuty = 0;   }

What this means in plain English:

Humidity (RH%)   Approx Fan %   Duty (0–255)   Description
≥ 82%             100%            255             Full power
≥ 80%             ~75%            191             Strong
≥ 78%             ~50%            128             Medium
≥ 75%             ~25%            64              Low/quiet
≥ 74%             ~10%            26              Minimum “breeze”
< 74%              0%             0               Fan OFF

⭐ How to adjust humidity activation points
You may change both the thresholds (RH%) and the duty values (0–255).
Example: Start the fan earlier at 70% humidity:
else if (h >= 70.0f) { humidityDuty = 26; }
Example: Make the 78% tier stronger:
else if (h >= 78.0f) { humidityDuty = 160; }

⭐ How to change “no-fan” zone
If you want the fan completely off below 72% instead of 74%:
else if (h >= 72.0f) { humidityDuty = 26; }
else                 { humidityDuty = 0; }

⭐ How to tune humidity sensitivity
Raise thresholds = fan runs less often.
Lower thresholds = fan activates sooner.
You can also simplify to a 3-speed system:
if (h >= 80.0f)      { humidityDuty = 255; }
else if (h >= 75.0f) { humidityDuty = 128; }
else if (h >= 70.0f) { humidityDuty = 64;  }
else                 { humidityDuty = 0;   }

⭐ Interaction with manual mode
Humidity control is disabled while manual fan mode is active.
Manual mode overrides:
• humidity thresholds
• minimum duty rules
• auto-off rules
When manual mode ends, humidity control resumes automatically.

⭐ Minimum Running Duty (stall protection)
Even if humidity wants a very low fan speed, the system prevents the fan from stalling:

const int FAN_MIN_RUN_DUTY = 25;

If your fan still stalls at low duty: increase FAN_MIN_RUN_DUTY.
If your fan works great at low speed: you can lower it.
Summary: fan speed increases with RH%, all values are editable, and minimum duty prevents stalling.

## 5) Wood Moisture Row

OLED example:
Wood Moisture: 11.2%
SM: -25488
The wood-moisture system is built around:
• ADS1115 ADC
• Differential electrode probe
• Median + EMA smoothing
• Calibration table with anchor shifting
• Nonlinear mapping from counts → % moisture

⭐ Where Moisture Is Read
Main read function:

float moistPct = readMoisturePctADS();

This returns the smoothed % moisture.

⭐ Probe Power (pulsed to prevent corrosion)

#define PROBE_PULSE 1

const int SENSOR_PWR_PIN = 2;

const unsigned long PULSE_SETTLE_MS = 300;

If PROBE_PULSE = 1, the probe is powered only during reads.
If you want the probe always powered, set PROBE_PULSE to 0.

⭐ Calibration Table (Counts → Moisture Mapping)
The code uses 8 calibration points:

const int16_t PT_COUNTS_BASE[NPTS] = {

-25503, -25500, -25497, -25489,

-25483, -25478, -25429, -24815

};

const float PT_PCT[NPTS] = {

0.0f, 2.5f, 5.0f, 8.0f,

9.0f, 12.5f, 14.5f, 17.0f

};

These describe what ADS1115 counts correspond to what % moisture.

⭐ Anchor Shifting (Your Custom Calibration)
You can shift the entire calibration curve up or down using the 2.5% anchor:

const float  USER_ANCHOR_PCT        = 2.5f;

int16_t      USER_ANCHOR_COUNTS_2P5 = -25500;

Changing USER_ANCHOR_COUNTS_2P5 shifts the whole curve.
Example:
If your wood reads wetter than reality:
USER_ANCHOR_COUNTS_2P5 = -25490;   // shifts curve upward (lower moisture %)
If readings look too dry:
USER_ANCHOR_COUNTS_2P5 = -25510;   // shifts curve downward (higher moisture %)

⭐ Smoothing Layers
1) Median filter (spike removal)

#define MEDIAN_WIN 11

Increase to 15 for extra stability; decrease to 7 for faster response.
2) EMA smoothing (gentle averaging):

#define EMA_ALPHA 0.15f

Smaller alpha → smoother, slower.
Larger alpha → more responsive but noisier.
Example:
#define EMA_ALPHA 0.25f   // quicker updates

3) Rate-limiter (prevents big jumps)
Inside readMoisturePctADS():

pct = clampDelta(pct, lastMoistPct, 0.20f);

This means moisture on the screen will change at most ±0.2% per sample.
Adjust to 0.10f for extra stability or 0.50f for more responsiveness.

⭐ Moisture Bar Graph Scaling

const float MOIST_BAR_MAX = 15.0f;

If you expect wetter wood, increase to 20.0f.
If always dry, reduce to 10.0f.

⭐ Ideal Range Markers

const float IDEAL_MC_LOW  = 8.0f;

const float IDEAL_MC_HIGH = 12.0f;

OLED shows two vertical ticks on the bar representing your ideal moisture window.
Adjust these to match your target range for your specific wood species.